Heather Hooper

Beam Reach – Fall 2007

September 11, 2007

Localization Exercise

On September 4, 2007 we played killer whale playback sounds from an underwater speaker and then recorded the sounds on a towed hydrophone array made up of four hydrophones.  The four hydrophones were connected to solid state recorders towed behind our research vessel, the Gato Verde, at known distances from the back left corner of the vessel.  Using the program Ishmael, the sound files can be analyzed by using the time difference between when the call reached each hydrophone to determine the origin of the call.  Each file was split up into one minute, 16bit files in order to make them easier to read.  After opening the file in Ishmael it displays a spectrogram given by each of the hydrophones on the main screen, this is shown in figure 1 for the playback file “709094_kwpb_s1_16bit.wav.”

Figure 1. Spectrograms in the Ishmael program of the first minute of killer whale playback recordings on four different hydrophones lined up in an array behind the research vessel.

There are four different ways to localize a call; phone-pair bearing, hyperbolic, cross-pair bearing, or beamforming.  We choose to use the hyperbolic localization option because it creates six hyperbola intersection points by pairing each of the hydrophones to each other to create the most accurate point of origin of the call.  Our speed of sound value is 1,985m/sec.  I used a grid map of 500m X 500m, because we most likely will not be localizing any calls further away than this.  We then choose a call to localize by drawing a box around the particular call and hitting the hyperbolic localization button.  Two screens then pop up.

One of the screens is cross-correlation graphs that compare the different hydrophone hyperbolas, and the point at which the calls match.  This is shown in figure 2, localizing the call at about 42 seconds into the recording.  The best correlation between the two hydrophones corresponds to the spike in the graph.  This basically gives you a qualitative confidence level in the accuracy of the localization of that call.  Since all six graphs show a high spike, then I can be relatively confident that the location Ishmael has computed is accurate.  If one or more of the graphs has a low spike or the line is rounded, then that may indicate a weak signal in one or more of the hydrophones.  This should be a warning that the source point location of the signal is less confident.

Figure 2. Cross-Correlation analysis of a localization call on four different hydrophones using the hyperbolic localization technique.

The other screen that pops up is the localization map, as seen in Figure 3.  This shows where the hydrophone array is located, given by our array document, and then the position of the localized call in relation to the origin.  At the top of this screen Ishmael also gives you the x and y coordinates of the origin of the call.  The coordinates of this call is (-27.94, 5.42) from the origin, which is fixed as the back left corner of the vessel.

Figure 3.  Localization map of a call, showing the hydrophone array (red dots), and the location of the call (yellow dot).

In addition, Ishmael spits out a log file in notepad that gives all of the hyperbolic equations and the final localization point of that call.  This can be very useful in keeping tract of the calls when localizing several calls within a file.

For my project I will be matching the localization of a call to the distance and bearing of individual whales while they are displaying surface behavior.  In order to do this I will have to determine the start time of the recorded file and match that to the real time of recorded behaviors of the whales.  The using the Pythagorean theorem we can calculate the distance between the origin and the call by calculating the hypotenuse of the right triangle.  Comparing the two values of bearing and distance with a certain level of error, should tell you that the whale and the call are associated.  Example calculations are shown below in Table 1.

| File: | 709094_kwpb_s1 &s2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Ishmael Loc |  |  |  |  |
| Real Time | Elapsed Time | X | Y | distance | bearing (radians) | bearing (degrees) |
| 18:22:14 | 0:00:14 | -61.53 | 108.18 | 124.4542 | -0.517142888 | -29.63010488 |
| 18:22:42 | 0:00:42 | -27.94 | 5.42 | 28.46085 | -1.379189015 | -79.02170973 |
| 18:23:21 | 0:01:21 | -58.04 | -32.03 | 66.2915 | -1.066525612 | -118.8925837 |
| 18:23:53 | 0:01:53 | -74.83 | -102.27 | 126.7229 | -0.631679761 | -143.8074157 |

| Boat Loc |  |  |
| --- | --- | --- |
| Record Time (sec) | Distance (m) | Bearing (degrees) |
| 19 | 126 | 25 |
| 33 | 40 | 97 |
| 124 | 73 | 130 |
| 153 | 112 | 150 |

Table 1.  Excel spreadsheet of four calculations comparing Ishmael localization, distance, and bearing with measurements of distance and bearing taken on the boat.

These excel spreadsheet values are a little off; this error is partly due to the differences in actual time between the call and the boat recording time.  In the future, I will work backwards, to determine which call to localize.  Ideally one that pertains more closely to the boat localization, while trying to take into account time the time lapse that is a result of slow reaction/writing time on the boat recordings.  It is also important to note that Ishmael does not distinguish between left and right signals, and so the positive and negative x values are not significant, only the absolute value.  Therefore, we must rely on our protractor bearings from the boat to tell if the vocalization was coming from the left or right side of the hydrophone.  As a general rule, any difference under 15 degrees will be considered relatively accurate because of the large degree of error when taking bearing measurements by sight with only the aid of a protractor.