WHALTRAK2 Program Documentation

by Jay Barlow

Program WHALTRAK2 was designed to plot positions of the ship and bearing angles to acoustic detections (such as sperm whale clicks or dolphin whistles) and to record other important data during an acoustic survey.    It receives serial input from the ship-board GPS (latitude and longitude) and from another PC computer running ISHMAEL (bearing angles to whale locations).    Whales are actually located when the plotted bearing angles converge on one location.      WHALTRAK2 is a Windows    program written in Intel Visual Fortran v.8.0 uisng QuikWin untilities.    It is a 2004 update on an earlier version of this program (WHALTRAK); all the previous features are preserved, but additional features have been added.    The program should run on all WinXP and Win98 operating systems.

The display in WHALTRAK2 shows a cartesian view of the ocean centered around the ship.    The concentric circles around the ship are one mile apart and provide a sense of scale.    The direction of the ship (from the past few GPS fixes) is displayed as a blue line.    Bearing lines to acoustic localizations are displayed as red lines.    The red lines will turn yellow after a fixed period of time (specified in the input file WHALTRAK2.INP), and will eventually disappear.    The previous path of the ship is indicated by a series of open blue circles.

Most commands in WHALTRAK are initiated by left-clicking the mouse cursor on a box along the left margin.    The box is left of the first letter of the following commands “Turn”, “Hydrophone Dist.”, “Comment”, “Operator”, and “Effort” (explained below).      Geographic scale is changed by left-clicking the up and down arrows in the upper left-hand corner.    The program is ended by left-clicking on the X in the upper right-hand corner or by using the “File/Exit” menu options.    Current cursor position (lat/long) and bearing and distance from the ship are displayed by left-clicking anywhere else in the plot space.    A red dot (marking a position) is added to the display by right-clicking anywhere in the plot space; the position of this dot is also recorded in the data file.

OUTPUT FILES

Commands (and bearing angles from the serial port) result in a data line being written to an output file called TRAKhhmm.mdd (where hhmm is start time hours and minutes and mdd is month and day).    An event code (in column 4 of the output files) indicates what data will follow the time, date, and position information.    In addition, a position is automatically written to the output file every X minutes, where X is specified by the parameter “mmin” in the input specification file (see below); the event code for automatic position updates is “*”.    An identical backup file “BAK_hhmm.mdd” is also written.    Both files are never open at the same time, so if, for any reason, data are lost from the main data file due to a computer crash, the data can be recovered from the backup file.    However, this must be done immediately.    If there are no problems, the BAK files can be deleted.

Effort: This command indicates whether you are searching acoustically for whales (with head-sets on) along a pre-determined transect line.    This button should be clicked “off” when you turn from the transect line to chase or localize whales, when you leave the station for any reason, or when the towed array is brought on-board.    Events codes are “R” for resume effort and “E” for end effort.    A period “.” is entered in column 5 of the output file for each on-effort event.

Operator: This is the first initials (typically 3 letters) of the operators name (in all capital letters).    The event code in the output file is “O”.

Hydrophone Distance: This is the separation distance between hydrophones (typically 3-7 meters).    Just enter the number, not the units (ie. just “3", not “3 m”).    This is for record-keeping only; you will still need to specify the hydrophone separation in ISHMAEL’s array file.    The event code in the output file is “H”.

Comments: This is to enter a comment up to 120 characters long.    Longer comments should be entered on multiple lines.    Liberal comments should describe anything you hear (including species, if known), anything seen by the visual observers and relayed to you, any significant changes in operating parameters (ship speed, wire-out, heading), etc.    Reasons should be given for going off effort.    Remember that your comments are the most important link to figuring out (at a later date) what was going on.    The event code in the output file is “C”.

Turn: This command indicates that the vessel is turning.    It changes the way that headings are calculated (averaging GPS readings over 1 minute rather than over 3 minutes as is done on straight-line courses).      WHALTRAK2 now has automatic detection of turns if the bearing angle changes more than 5 degrees between fixes.    The automatic timed position events (*) will be at 1-minute intervals during turns.    The event code in the output file is “T”.

Input Bearing Angles:    Normally, bearing angles are input from ISHMAEL via the serial port (9600 baud, 8 bits, 1 stop bit, no parity).    These data consist of a bearing angle (in degrees) preceded by a $ character and followed by a # character.    Bearing angles can also be entered from the keyboard by clicking the box next to “Input Bearing”.    Values can be between zero and 360 degrees.    Both left and right bearing angles will be plotted.    The event code in the output file is “D”.

Click Timer: The period between events (such as sperm whale clicks) can be entered by clicking the box next to “Click Timer”.    The first time will act as a reference.    Each time after that, the interval between clicks will be recorded and displayed.    Comments should be entered to indicate what you are timing.    The event code in the output file is “I”.

Erase Lines After: You can change the time parameter that determines how long lines will persist on the screen.    You can use this to clear the screen of excess clutter from previous bearing angles.    Lines are not actually erased and can be displayed again by increasing the time parameter.

Start/End Recording:  Enter the start and end times for tape or hard-disk recordings.    You should enter “START” or “END” and the tape number or recording type (eg. HD).    For tapes, you should also enter the elapsed time.      The event code in the output file is “r”.

Timed Updates (5-minute updates): Enter codes for what you have heard over the last 5 minutes (such as DW34).    The event code in the output file is “U”.

Log File:

A separate log file (“LOG_hhmm.mdd”) is written that includes the time, position, heading and speed at much more frequent intervals (set by the parameter “msec” in the input file).    This file may be useful to re-construct the details of a cruise track.

WHALTRAK Operational Modes:	WHALTRAK can be run in several modes.    Operational mode is changed in the input configuration file (WHALTRAK2.INP), and cannot be changed while the program is running.

Normal Real-time Display-

WHALTRAK2 will receive GPS input and plot ship tracks and bearing angles.    Optionally, can run GSP Diagnostics.    This is the default if “Simulation” and “Replay” are both FALSE in the input file.    You will need two COM ports on your computer to run in real-time mode (or you will need to enter bearing angles by keyboard).    On older laptops, you can add COM ports with PCMCIA cards.    With new laptops, you can get USB COM ports (like KeySpan products).

Simulation-

WHALTRAK2 will simulate a GPS input, which is useful for testing or training if you are not connected to a GPS or are not moving.    The ship will travel due North starting at Lat=0 and Long=0 at a speed that you specify in the input file.    The program will still receive bearing angle    input from ISHMAEL (if connected).    “Simulation” should be set to TRUE in the input file.

Replay-

WHALTRAK2 will replay and re-plot a previously TRAK file.    The program will prompt you for a file name and an end time.    Only data prior to that end time will be plotted.    This is useful to print plots of TRAK events after a cruise is over.    “Replay” should be set to TRUE in the input file.

Configuration Input File:

A configuration file called WHALTRAK2.INP is required to run WHALTRAK2.    It must be in the same directory or folder.    An example is:

1       !COM PORT FOR ISHMAEL INPUT OF BEARING ANGLES (1, 2, 3, OR 4;  HIGHER VALUES MAY WORK)

2       !COM PORT FOR GPS INPUT (1, 2, 3, OR 4)  (MUST BE DIFFERENT FROM ISHMAEL INPUT)

10      !MSEC= SECONDS BETWEEN POSITION UPDATES ON SCREEN (MAX= 30)

5       !MMIN= MINUTES BETWEEN POSITION WRITE_TO_FILE (MAX= 30)

20      !REDTIME= MINUTES BEFORE BEARING LINES WILL TURN FROM RED TO YELLOW

30      !DEADTIME= MINUTES BEFORE LINES WILL STOP PLOTTING

300     !DISTARAY= DISTANCE FROM gps TO CENTER OF ARRAY IN METERS

GLL     !GPS line code specifying latitude and longitude information

'(I2,F6.3,1X,A1,1X,I3,F6.3,1X,A1)'     !GPSFORMAT (LATDEG,LATMIN,NORS,LONGDEG,LONGMIN,EORW)

FALSE                                  !PRINT GPS DIAGNOSTICS TO GPS.ERR FILE (T OR F)

FALSE	10.0                          !SIMULATE GPS INPUT (T or F) AND SIMULATED SPEED IN KTS

FALSE                                  !REPLAY A WHALTRAK FILE (T or F)

!The following lines are comments and are not read by the program WHALTRAK2.

SOME COMMON GPS FORMATS (COPY INTO APPROPRIATE POSITION ABOVE:

GLL

'(I2,F5.2,1X,A1,1X,I3,F5.2,1X,A1)'		!GPSFORMAT USING $GPGLL LINE, XX.XX DECIMAL MINUTES

GLL

'(I2,F6.3,1X,A1,1X,I3,F6.3,1X,A1)'		!GPSFORMAT USING $GPGLL LINE, XX.XXX DECIMAL MINUTES

This file is a simple ASCII formatted file.    It must be editted with a ASCII editor, such as Notepad or VEDIT.    Data must be entered in the exact format given above.    Information to the left of the “!”marks are comments for explanation only.

GPS Input    - GPS input to WHALTRAK2 is via a serial port (COM# is specified in WHALTRAK2.INP).    The input standard is NMEA183 (4800 baud, 8 bits, 1 stop bit, no parity), however, that does not fully specify what data lines are outputted.    Position information is commonly available on the $GPGLL data line, but may appear on different data lines from different GPS units.    Also, the number of digits in the decimal minutes varies between GPS units (typically 2 or 3 digits to the left of the decimal point).    If you have a problem with receiving GPS data, start WHALTRAK2 in GPS diagnostic mode (change TRUE to FALSE in the appropriate line in the file above).    After it has run in diagnostic mode for a minute, you can look at the GPS.ERR file to see the data that have been received from the GPS port.    If the GLL line is unavailable, you can chose another output line type.    Copy the appropriate 3-letter code and corresponding    format statement into the appropriate line of the WHALTRAK.INP file.    Remember that this file is a simple ASCII file, and the lines and data must appear in the exact order given above.    The format statement follow FORTRAN standards, so you may be able to find an old FORTRAN programmer to help you figure this out if you have problems.    If you email a small GPS.ERR file (in GPS diagnostic mode) to me at Jay.Barlow@noaa.gov, I may be able to quickly figure out a problem.

Do not routinely run in GPS diagnostic mode or the program may be significantly slower in operation and response.    The GPS.ERR file will also grow to be enormous.